District:   ________________________________________________________________________________

School:  _________________________________________________________________________________

Play Site Location:   ________________________________	Age of Intended User:  _________________

Equipment Manufacturer:  __________________________________________________________________

Equipment Installer:  _______________________________________________________________________

Date of Inspection:   ________________________________________________________________________

Inspector(s):  ______________________________________________________________________________

| C | NC | N/A | SURFACING MATERIAL | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Surfacing material is free of debris, weeds, and standing water. CPSC 2.4.2 |  |
|  |  |  | Area is free of trip hazards |  |
|  |  |  | All anchoring posts and devices are below material |  |
|  |  |  | Loose-fill surfacing material: |  |
|  |  |  | Fall material is a minimum of 9 inches deep CPSC 2.4.2.2 |  |
|  |  |  | Fall material is evenly distributed (i.e. it is not piling up under platforms, etc.) CPSC 2.4.2.2 |  |
|  |  |  | Fall material is loose and ample CPSC 2.4.2.2 |  |
|  |  |  | Unitary surfacing material: |  |
|  |  |  | Surfacing material is in good condition |  |

Type of fall material:    ____________________________________________________________________________

ASTM F1292-99/04, F1951-99 and F2075 (if engineered wood fiber) certifications/test results on file: ____________

Playground documentation file created (instructions, warranty/liability info, EWF installation instructions: ________

Additional comments regarding fall material:  _________________________________________________________

_______________________________________________________________________________________________

| C | NC | N/A | USE ZONES | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Use zone around stationary equipment is 72” ASTM 9.2.1 - Adjacent play structures - 30” or less can be 6’; more than 30” must be 9’ ASTM 9.2.3 |  |
|  |  |  | Use zone in front of slides is minimum 6’ – maximum 8’ ASTM 9.6.2.1 - Slides greater than 6 feet, use zone in front should be as long as the slide is tall up to a max of 8 feet. CPSC 5.3.6.5 - Use zone shall never overlap another use zone. CPSC 5.3.6.5 |  |
|  |  |  | Use zone for rotating play equipment is 72” ASTM 9.3.1 - Use zone may not overlap any other equipment ASTM 9.3.2 |  |
|  |  |  | Use zone for swings is twice the height of surface material to the pivot point ASTM 9.4.1.1 - Front-to-rear use zone cannot overlap any other equipment. CPSC 5.3.8.3.1 - Use zone 6 feet from support posts CPSC 5.3.8.3.3 - Use zone of support structure for adjacent swings may overlap ASTM 9.4.1.5 (1) |  |
|  |  |  | Use zone for bucket seat swings is twice the height from seat to the pivot bar ASTM 9.4.1.2, CPSC 5.3.8.3.3 - Use zone 6 feet from support posts CPSC 5.3.8.3.3 |  |
|  |  |  | Use zone for multi-axis swings is the height of the swing seat to the pivot point plus 72” ASTM 9.4.1.2 - Use zone 6 feet from support posts CPSC 5.3.8.4 - Use zone cannot overlap any other equipment CPSC 5.3.8.4 - Use zone of support structure for adjacent swings may overlap ASTM 9.4.2.5, CPSC 5.3.8.4.1 |  |
|  |  |  | Use zone for rocking/springing play equipment - 72” if user is intended to sit ASTM 9.5.1.1 - May overlap when each structure consists of a seat with height of 30” or less ASTM 9.5.1.2 |  |
|  |  |  | Use zone for rocking/springing play equipment - 84” if user is intended to stand ASTM 9.5.2.1 - Use zone may not overlap any other equipment ASTM 9.5.2.2 |  |
|  |  |  | Use zone for a track ride shall extend no less than 72” in all directions ASTM 9.9.1 |  |
|  |  |  | Overhead obstructions within the use zones are 84” above designated playing surfaces and pivot of swings. ASTM 8.14.1 (roofs) and ASTM 9.8.4.1 (Also Roof 8.14.2 – if less than 84”, does not contain designated play 2”x2”flat surface) |  |

| C | NC | N/A | ACCESSIBILITY | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Accessible route outside and access point into play area - 60” minimum clear width w/ designated entrance 2016 CBC 11B-1008.2.4.1 - 44” minimum clear width if play area less than 1000 sq. feet. - Exterior walkway may be narrowed to 36” for a distance of 60” (i.e. tree narrows path) - 80” vertical clearance along ground level accessible route 2016 CBC 11B-1008.2 - No change in level greater than ¼” vertical or ½” beveled along path of travel at ramp and entrance to the play area. 2016 CBC 11B-1008.2 and 11B-303 - Maximum entrance ramp slope is no more than 5.0% (1:20) into the play area w/o handrails. 2016 CBC 11B-403.3 With handrails 6.25% (1:16) 2016 CBC 11B-1008.2.5 |  |
|  |  |  | Accessible route/clear ground space inside play box - 60” minimum clear width to transfer platform and each ground level play component 2016 CBC 11B-1008.2.4.1 - Minimum 30” wide x 48” long clear ground space positioned for forward or parallel approach to the ground level play component 2016 CBC 11B-1008.4.2 - Accessible elevated play component route – minimum 36” wide, can be reduced to minimum 32” for a distance of 24” 2016 CBC 11B-1008.2.4.2 - Minimum 24” allowable at transfer steps |  |
|  |  |  | Transfer platform - 11” to 18” above the protective surfacing - Minimum 14” deep x 24” wide 2016 CBC 11B-1008.3.1.2 - Transfer support provided 2016 CBC 11B-1008.3.1.4 - Minimum 48” x 48” transfer space area adjacent to and centered on the platform 2016 CBC 11B-1008.3.1.3 - May be primary access if play area has less than 20 elevated play components 2016 CBC 11B-1008.2.1 |  |
|  |  |  | Transfer steps: - 24” wide minimum 2016 CBC 11B-1008.3.2.1 - 14” deep minimum - 8” high maximum 2016 CBC 11B-1008.3.2.2 - Contrast striping on upper approach and each stair tread - full width 2-4” wide stripe, within 1” maximum from front edge. Paint is acceptable 2016 CBC 11B-1008.3.2.4 and 11B-504.4.1 |  |
|  |  |  | Entry point/seats on play equipment (i.e. swings, play bench, any element requiring transfer to a seat) - 11” to 24” to top of seat above protective surfacing 2016 CBC 11B-1008.4.4 - Swings only - Minimum 60” circle or T-shaped turning space located immediately adjacent to swing 2016 CBC 11B-1008.4.1 |  |
|  |  |  | Play tables: (tables for 2-5 may be parallel approach) - 24” high minimum knee clearance 2016 CBC 11B-1008.4.3 - 31” maximum height of table - 30” wide minimum - 17” deep minimum |  |  |
|  |  |  | Required # of accessible play components: |  |  |
|  |  |  | - Ground level (see Table A for appropriate number. See Table B for recommended reach range to highest operable part of play element.) 2016 CBC 11B-240.2.1 |  |  |
|  |  |  | - Elevated – at least 50% of the total elevated play components must be on an accessible route from the transfer platform 2016 CBC 11B-1008.2.2 |  |  |

| TABLE A: Required Number of Play Components |  |  |
| --- | --- | --- |
| Number of elevated play components provided | Minimum number of ground-level play components required to be on accessible route | Minimum number of different types of ground-level play components required to be on accessible route |
| 1 | Not applicable | Not applicable |
| 2 to 4 | 1 | 1 |
| 5 to 7 | 2 | 2 |
| 8 to 10 | 3 | 3 |
| 11 to 13 | 4 | 3 |
| 14 to 16 | 5 | 3 |
| 17 to 19 | 6 | 3 |
| 20 to 22 | 7 | 4 |

| TABLE B: Recommended Reach Ranges |  |  |  |  |
| --- | --- | --- | --- | --- |
| Forward or Side Reach | Ages 3 and 4 | Ages 5 through 8 | Ages 9 through 12 | Compliance Requirement |
| High (maximum) | 36 inches | 40 inches | 44 inches | 48 inches |
| Low (minimum) | 20 inches | 18 inches | 16 inches | 15 inches |

|  |  |  | If playground has 20 or more elevated play components: |  |
| --- | --- | --- | --- | --- |
|  |  |  | Elevated accessible routes: - Ramp to elevated play components required - Minimum 36” wide ramp width - Ramp - handrail height between 20” and 28” - Maximum 12” ramp rise |  |
|  |  |  | Landings - Minimally as wide as the ramp at bottom and top landing - Top landing - minimum length 60” to allow for direction change - Minimum 60” turning space |  |

| C | NC | N/A | STAIRWAYS, LADDERS AND HANDRAILS | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Rung ladders, climbing nets, arch climbers, tire climbers are not used as the sole access on equipment intended for pre-school age children ASTM 7.3.2.1 - Flexible access components for 2-5 years allows user to bring both feet to same level before ascending to next level ASTM 7.3.2.4 |  |
|  |  |  | Flexible components used for access are securely connected at both ends ASTM 7.3.2.2 - Anchoring device must be beneath the ground surface material ASTM 7.3.2.2, CPSC 5.3.2.3 |  |
|  |  |  | Climbers used for access have hand support for use while climbing ASTM 7.3.2.5, CPSC 5.2.4 |  |
|  |  |  | Rungs and handrails have a diameter between .95 and 1.55 ASTM 7.2.6.4 - Handgrips are secured to prevent turning CPSC 5.2.2 - Between .60” and 1.20” for toddlers CPSC 5.2.2 |  |
|  |  |  | The stepping surface used for final access (for rung ladders, flexible components, arch climbers, etc.) is not above the designated play surface it serves. ASTM 7.4.3 |  |
|  |  |  | Handrail height: Vertical distance between the edge of the step and the top surface of the handrail is (CPSC 5.2.3.1): - Between 15” and 20” for toddlers - Between 22” and 26” for preschool age - Between 22” and 38” for school age - Begins with the first step Vertical infill for protective barriers may be preferable for younger children because the vertical components can be grasped at whatever height a child chooses as a handhold |  |
|  |  |  | All steps greater than 48 inches above the protective surfacing have a protective barrier instead of a handrail. ASTM 7.5.6.1 (1) |  |
|  |  |  | PLATFORMS |  |
|  |  |  | Maximum difference in height between stepped platforms are (ASTM 7.5.7.1) - 7” for toddlers - 12” for preschool age children - 18” for school age children - If difference is more than noted, an access component is needed CPSC 5.1.2 |  |
|  |  |  | Openings between platforms are not a head entrapment hazard CPSC 5.2.1 |  |
|  |  |  | If the space between platforms exceeds 9” and the height of the lower platform exceeds 30” for preschool or 48” for school age, in-fill is used to reduce the space to less than 3.5” CPSC 5.1.2 |  |
|  |  |  | Platforms intended for toddlers are no more than 32” above the protective surfacing CPSC 5.1.1 |  |

| C | NC | N/A | GUARDRAILS AND PROTECTIVE BARRIERS | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Openings between uprights and between barrier and platform surface are not a head entrapment hazard CPSC 5.2.4 |  |
|  |  |  | Guardrails and barriers (CPSC 5.1.3): - Designed to prevent falls off the platform - Discourage climbing on the barrier - Aid supervision |  |
|  |  |  | Guardrails/barriers completely surround elevated platform except for entrance and exit opening (ASTM 7.5.5.2 (2) , ASTM 7.5.6.3 (3), CPSC 5.1.3) - 15” maximum to access play event - If more than 15”, must have one top rail of a guardrail - Stairs, ramps and upper body equipment are exempt from this requirement |  |
|  |  |  | Solid panels used as infill have transparent areas to facilitate supervision and permit viewing from platform. CPSC 5.1.3 |  |
|  |  |  | Guardrails are provided for preschool age (ASTM 7.5.5.1, 7.5.5.3, 7.5.5.4): - Elevated surface more than 20” but not over 30” above protective surface - Minimum height of top rail is 29” - Maximum height of lower rail is 23” |  |
|  |  |  | Guardrails are provided for school age (ASTM 7.5.5.1, 7.5.5.3, 7.5.5.4): - Elevated surface more than 30” but not over 48” above protective surface - Minimum height of top rail is 38” - Maximum height of lower rail is 28” |  |
|  |  |  | Barriers are provided for toddlers (CPSC 5.1.3): - Elevated surface more than 18” above protective surface - Minimum height of barrier is 24” - Guardrails are not recommended |  |
|  |  |  | Barriers are provided for preschool age (ASTM 7.5.6.1, ASTM 7.5.6.4): - Elevated surface more than 30” above protective surface - Minimum height of barrier is 29” |  |
|  |  |  | Barriers are provided for school age (ASTM 7.5.6.1, ASTM 7.5.6.4): - Elevated surface more than 48” above protective surface - Minimum height of barrier is 38” |  |

| C | NC | N/A |  | DURABILITY & FINISH |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Metal surfaces painted or treated to prevent rust (CPSC 2.5.3): - Bare metal slides or platform surfaces are to be avoided unless away from the sun or covered |  |  |
|  |  |  |  | Painted surfaces are in good shape and not chipped CSPC 2.5.4 |  |  |
|  |  |  |  | Wood structures or play elements are in good condition ASTM 4.1.3, CSPC 2.5.5 - Wood support posts are undamaged below fall material. |  |  |
|  |  |  |  | Plastic materials and components are in good condition ASTM 4.1.1 |  |  |
|  |  |  |  | HARDWARE |  |  |
|  |  |  |  | All nuts, bolts, pins, lock washers and other connectors are present and cannot be loosened or removed without the use of a tool ASTM 4.2.2, CPSC 2.5.2 |  |  |
|  |  |  |  | All posts are capped and sealed CPSC 2.5.2 |  |  |
|  |  |  |  | Hardware in moving joints are secure CPSC 2.5.2 |  |  |
|  |  |  |  | All S-hooks are closed so there is no gap or space greater than .04” or wide enough to admit a dime. CPSC 2.5.2 |  |  |
|  |  |  |  | PROJECTIONS, PROTRUSIONS AND ENTANGLEMENTS |  |  |
|  |  |  |  | Equipment is free of sharp points and corners are rounded. Caps or plugs cover exposed open ends of tubing. Wood parts are smooth and free of splinters. CPSC 3.4 |  |  |
|  |  |  |  | Protrusions and projections do not extend beyond the face of any of the 3 gauges nor increase in the direction away from the surrounding surface nor project beyond the face of the nut more than 2 full threads. ASTM 6.4.3, ASTM 6.3.2, CPSC 3.2 |  |  |
|  |  |  |  | Equipment is free of entanglements. A projection is not an entanglement hazard unless the following are present (ASTM fig A1.15): - Projection must fit within a projection gauge. - Projection must be above the horizontal plane. - Projection must have perpendicular sides. - Projection must extend more than 1/8” from initial surface |  |  |
|  |  |  |  | Recessed bolts are not entanglement or protrusion hazards. ASTM 6.4.3 - If the curved surface of the gauge touches the bolt, it is subject to the protrusion tests. |  |  |
|  |  |  |  | Projections do not increase in size from their initial surface more than 1/8” in width and depth creating an entanglement hazard. ASTM 6.4, CPSC 3.2 |  |  |
|  |  |  |  | Protrusions on swing assemblies (CPSC 5.3.8.5): - Using the swing test gauge, no bolt or component protrudes beyond it (1/8”). |  |  |
| C | NC | N/A |  | PROJECTIONS, PROTRUSIONS AND ENTANGLEMENTS (CONT.) |  | Comments |
|  |  |  |  | Connecting devices on swings (ASTM 6.4.5.1, CPSC 5.3.8.1): - S-hooks, C-hooks, pelican hooks, etc. are closed so there is no gap or space greater than .04” - No portion of the lower loop of an S-hook projects beyond the vertical boundary of the upper loop - Upper loop of S-hook may align, partially overlap or completely overlap without extending past the connector body - S-hook lower loop aligns with connector body and does not overlap. |  |  |
|  |  |  |  | Protrusions on slides : - Not greater than 1/8” - Slide has a smooth, continuous sliding surface without any gaps or spaces that may create an entanglement hazard ASTM 6.4.1.2 |  |  |
|  |  |  |  | No accessible crush or shear points which may be caused by components moving relative to each other or to a fixed component - Roller slides – use 3/16” dowel - Merry-Go-Rounds – use 5/16” dowel (test gap between understructure and top surface) - Common crush/shear – use 5/8 “ dowel (Section 5 item #1). ASTM 6.5, 8.9.2, 8.8.4.1; CPSC 3.1 - Bridges – test between stationary platform and first moving board |  |  |
|  |  |  |  | LABELS |  |  |
|  |  |  |  | Signs and/or stickers indicate the following: |  |  |
|  |  |  |  | - age appropriateness of the equipment ASTM 14.2.1, CPSC 2.2.6 |  |  |
|  |  |  |  | - supervision recommendation ASTM 14.2.2, CPSC 2.2.7 |  |  |
|  |  |  |  | - hazard of play equipment located over hard surfaces ASTM 14.2.5 |  |  |
|  |  |  |  | - equipment manufacturer identified ASTM 15.1 |  |  |
|  |  |  |  | - hot play surfaces warning ASTM 14.2.4 |  |  |
|  |  |  |  | - hazards of drawstrings, accessories and other things worn around the neck ASTM 14.2.3 |  |  |
|  |  |  |  | - removal of helmets (i.e. bike helmets) ASTM 14.2.3 |  |  |
|  |  |  |  | PARTIALLY BOUND OPENINGS AND ANGLES |  |  |
|  |  |  |  | Partially bound openings do not present an entrapment hazard ASTM 6.1.4.1 - Use the partially bound opening template (“fish probe”) - Partially bound openings less than 24” above the surface are exempt ASTM 6.1.4.7 (3) |  |  |
|  |  |  |  | Angles formed by adjacent components are greater than 55 degrees CPSC 3.3.2 - Unless lower leg is horizontal or projects downward |  |  |
| C | NC | N/A | CLIMBING AND UPPER BODY EQUIPMENT |  | Comments |  |
|  |  |  | Maximum fall height for free standing and composite climbing structures for toddlers is 32” CPSC 5.3.2.1.2 |  |  |  |
|  |  |  | Climbers do not have bars or other structural components in the interior of the structure onto which a child may fall from a height of more than 18” CPSC 5.3.2.1.5 |  |  |  |
|  |  |  | Equipment allows children to descend as easily as they ascend. Especially important for preschool equipment. CPSC 5.2.1 |  |  |  |
|  |  |  | Layout of equipment does not facilitate climbing to top support bars of upper body equipment nor interfere with movement on adjacent structures. CPSC 5.3.2.1.1 |  |  |  |
|  |  |  | Spacing between components does not present head entrapment hazards CPSC 5.2.4 |  |  |  |
|  |  |  | Flexible climbing equipment: - Is not the sole means of access to components intended for use by 2-5 year olds or toddlers ASTM 7.3.2.1, CPSC 5.3.2.3 - Anchoring device is beneath surfacing CPSC 5.3.2.3 - Perimeter of net opening should be less than 17” or greater than 28” CPSC 5.3.2.3 - Head probe cannot be pushed through opening with up to 50 lbs. of force ASTM 6.1.2 - Flexible components (i.e. rope, cable or chains) are secured at both ends and not capable of being looped back on itself. ASTM 7.3.2.2 - Flexible components suspended between play units are not located in high traffic areas. (Exception: 84” high, min 1” wide). - Free standing flexible climbers are not recommended for toddlers or preschool age CPSC 5.3.2.3 |  |  |  |
|  |  |  | 3D Climbing Nets: - No clear opening between flexible members with a vertical dimension greater than 72” and a diameter greater than (ASTM 8.2.5.1): - 18” for nets intended for 2-5 year olds - 20” for nets intended for 5-12 year olds |  |  |  |
|  |  |  | 3D Climbing Nets: - Minimum fall height for structure greater than 72” high is 72” ASTM 8.2.5.1 - Exterior fall height – distance from surfacing to highest point at which a rigid vertical device contacts the net structure when moved around the perimeter. ASTM 8.2.5.1 (1) - Interior fall height – distance from surfacing to the highest member with a clear vertical path to the surfacing with a diameter of 18” for 2-5 years old and 20” for 5- 12 years old. ASTM 8.2.5.1(2) |  |  |  |
| C | NC | N/A | CLIMBING AND UPPER BODY EQUIPMENT |  | Comments |  |
|  |  |  | Arch climber (CPSC 5.3.2.2): - Is not the sole means of access to components intended for use by 2-5 year olds - Free standing arch climbers are not allowed on preschool or toddler playgrounds - Rungs are .95” to 1.55” in diameter CPSC 5.2.2 |  |  |  |
|  |  |  | Horizontal ladders and overhead rings (CPSC 5.3.2.4): - When access is by rung, the first handhold must be between 8-10 inches from the access rung. ASTM 8.3.2 - When access is by platform, the first handhold must be between 0-10 inches from the leading edge of the platform. ASTM 8.3.2 - Rungs on ladders intended for 4-5 year olds are parallel and evenly spaced - Space between adjacent rungs – no more than 12” for preschool and 15” for school age (does not apply to rings if rings swing) - Maximum length of chains for overhead rings is 7 inches. ASTM 8.3.5, CPSC 5.3.2.5 - Moveable rings - Maximum 15” from pivot to bottom of handgrip ASTM 8.3.5 |  |  |  |
|  |  |  | Horizontal ladders and overhead rings: - Maximum height from center of grasping device to surfacing is 60” for preschool or 84” for school age ASTM 8.3.3, CPSC 5.3.2.5 - Maximum height for take-off/landing structure is 18” for preschool and 36” for school age ASTM 8.3.4, CPSC 5.3.2.5 |  |  |  |

| C | NC | N/A | EQUIPMENT | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Balance beams (ASTM 8.1.1, CPSC 5.3.1): - Maximum height 12” for preschool - Maximum height 16” for school age - Not recommended for toddlers |  |
|  |  |  | Climbing ropes (ASTM 6.6.1, CPSC 3.5): - Secure at both ends and not able to make a loop with an inside perimeter greater than 5” (diameter of 1.6 inches) |  |
|  |  |  | Log rolls (CPSC 5.3.3): - Not Recommended for use by children less than 5 years ASTM 8.12.2 - 18” above fall surface material ASTM 8.12.3 - Hand gripping components for use; .95” to 1.55” in diameter ASTM 8.12.3 |  |
|  |  |  | Track Rides (CPSC 5.3.2.7): - Not Recommended for use by children less than 5 years ASTM 8.13.1 - Hand grip is 64” minimum and 78” maximum above the surface material ASTM 8.13.2 - Landings (if used) have a minimum 36” length and 32” minimum width ASTM 8.13.3 - Center to center distance between adjacent tracks are 48” minimum ASTM 8.13.6 |  |
|  |  |  | Seesaws: - Not Recommended for toddlers and preschool unless equipped with a spring centering device ASTM 8.10.1, CPSC 5.3.5.2 - Fulcrum is free of pinch and crush points ASTM 8.10.2, CPSC 5.3.5.1 - Shock absorbing material under seats to minimize impact with ground ASTM 8.10.2 - Handholds do not protrude beyond sides of seat ASTM 8.10.4.1, CPSC 5.3.5.3 - Handgrips are 3” minimum if intended to be gripped by one hand and 6” if intended to be gripped by 2 hands ASTM 8.10.4.1 - Maximum 60” height of seat above protective surface ASTM 8.10.6 |  |
|  |  |  | Sliding poles (CPSC 5.3.2.6): - Not Recommended on equipment for toddlers and preschool age - Continuous with no welds or seams ASTM 8.4.5 - Horizontal distance between platform edge and pole is between 18” and 20” ASTM 8.4.1 - Pole extends at least 60” above the platform ASTM 8.4.3 - Pole diameter is 1.9” maximum ASTM 8.4.4 - Guardrail or barrier at pole access has an opening of 15” maximum ASTM 8.4.6 |  |
| C | NC | N/A | EQUIPMENT | Comments |
|  |  |  | Stepping Forms: - Minimum designated play surface is 10” in diameter. ASTM 8.15.1 - Maximum height to protective surfacing is 20” for preschool or 30” for school age unless hand support is present. ASTM 8.15.3 - Hand supports are required if the first stepping form height is greater than 20” for preschool or 30” for school age. The hand support height should be 22”-38”; protective surfacing to top of hand support. ASTM 8.15.4 - Stepping forms used by 2-5 year olds or more than 30” above protective surfacing must be stationary. ASTM 8.15.5 - Maximum distance between stepping forms are 12” for preschool or 18” for school age. Measurements are done edge to edge. ASTM 8.15.6 |  |

| C | NC | N/A |  | SLIDES |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Depth of transition platform is 14” or greater (ASTM 8.5.3.2.2, CPSC 5.3.6.2): - Width equal to or greater than width of sliding chute ASTM 8.5.2.3 - Depth for equipment intended for toddlers is 19” |  |  |
|  |  |  |  | Guardrail or hood present to channel user into a sitting position (ASTM 8.5.3.2, CPSC 5.3.6.2) |  |  |
|  |  |  |  | Straight slides with flat open chutes have 4”minimum sides for entire length of sliding surface (ASTM 8.5.4.4, CPSC 5.3.6.3.4) |  |  |
|  |  |  |  | Slide chute surface shall not exceed a height/length ratio of 0.577 (Height of slide divided by length) with no slide surface span exceeding 50°. (ASTM 8.5.4) |  |  |
|  |  |  |  | Slide chute is continuous without any gaps. Roller slides are excluded from this requirement. (CPSC 5.3.6.2) |  |  |
|  |  |  |  | 21” of clearance surrounds the slide chute (ASTM 9.6.3) |  |  |
|  |  |  |  | Metal slides are shaded to prevent burns from direct sun (ASTM 2.5.3) |  |  |
|  |  |  |  | Exit region of slide is (CPSC 5.3.6.4): - 11” above protective surfacing for slides no more than 4 feet in height ASTM 8.5.5.3 - Between 7” and 15” above protective surfacing for slides over 4 feet in height ASTM 8.5.5.3 - Toddler slides – no more than 6” above protective surfacing |  |  |
|  |  |  |  | Embankment slides (CPSC 5.3.6.3.1): - Maximum height of 12” above the ground - Chute entrance minimizes use by skateboards, bikes, etc. |  |  |
|  |  |  |  | Roller slides (ASTM 8.9.2, 8.9.2.1, CPSC 5.3.6.3.2): - Space between adjacent rollers and between ends of rollers and stationary structure are less than 3/16” (smallest dowel) |  |  |
|  |  |  |  | Spiral slides (CPSC 5.3.6.3.3): - One turn (360 degrees) or less for toddlers and preschool |  |  |
|  |  |  |  | Tube slides (CPSC 5.3.6.3.5): - Minimum internal diameter no less than 23” - Barriers or such should be considered to prevent climbing on top of the outside of the tube. |  |  |
| C | NC | N/A | SWINGS |  | Comments |  |
|  |  |  | Prohibited: - Swings attached to composite structures ASTM 8.6.1.2, CPSC 5.3.8.3.1 - Animal figure swings CPSC 2.3.1 - Rope swings CPSC 2.3.1 - Swinging dual exercise rings and trapeze bars CPSC 2.3.1 |  |  |  |
|  |  |  | S-hooks are closed with no gap or space great than .04”. Gap cannot admit a dime. ASTM 6.4.5, CPSC 5.3.8.1 |  |  |  |
|  |  |  | Swing structure discourages climbing. A-frame supports do not have cross-bars ASTM 8.6.2, CPSC 5.3.8.1 |  |  |  |
|  |  |  | 2 swings per bay maximum ASTM 8.6.4.4, CPSC 5.3.8.3.1 |  |  |  |
|  |  |  | Swing hangers at pivot spaced no less than 20” apart ASTM 8.6.5.1 (4) |  |  |  |
|  |  |  | **Swing spacing – toddler (bucket): - 20” between swing and support structure - 20” between swings - No less than 24” underside of occupied swing to surface ASTM 8.6.5.1 (5) - Located in a separate bay from other swings CPSC 5.3.8.3.2 - Pivot points are between 47” and 96” above the protective surfacing CPSC 5.3.8.3.2 |  |  |  |
|  |  |  | **Swing spacing – preschool: - 30” between swing and support structure ASTM 8.6.5.1 (3) - 24” between swings ASTM 8.6.5.1 (2) - No less than 12” underside of occupied swing to surface ASTM 8.6.5.1 (5) - ADA accessibility – entry point of seat between 11” and 24” above surfacing material |  |  |  |
|  |  |  | **Swing spacing – school age: - 30” between swing and support structure ASTM 8.6.5.1 (3) - 24” between swings ASTM 8.6.5.1 (2) - No less than 12” underside of occupied swing to surface ASTM 8.6.5.1 (5) - ADA accessibility – entry point of seat between 11” and 24” above surfacing material |  |  |  |
|  |  |  | Multi-Axis Tire Swing (CPSC 5.3.8.4): - Minimum clearance between seating surface and upright of support structure is 30” when tire is in a position closest to the support structure. ASTM 8.6.5.3 (2) - No less than 12” underside of occupied swing to surface ASTM 8.6.5.3 (1) |  |  |  |

** Measurements taken at 60” above protective surfacing

| C | NC | N/A | ROTATING & ROCKING EQUIPMENT | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Merry-Go-Rounds and Whirls: - Components do not extend beyond the perimeter of the platform ASTM 8.8.2, CPSC 5.3.4 - Underside of platform is no less than 9” above the surface ASTM 8.8.4.2, CPSC 5.3.4 - No openings between the axis and periphery that permit a 5/16 rod to penetrate ASTM 8.8.4.1, CPSC 5.3.4 - No accessible shearing or crushing mechanism in the undercarriage CPSC 5.3.4 - Maximum height for standing/sitting surface is 14” above surface for preschool and 18” for school age ASTM 8.8.2, CPSC 5.3.4 - Hand grips have a diameter of 0.95” to 1.55” CPSC 5.2.2 - Not recommended for playgrounds intended for toddlers CPSC 5.3.4 |  |
|  |  |  | Spring Rockers: - Springs minimize possibility of pinching hands or feet ASTM 8.11.4, CPSC 5.3.7 - Each seating position has handgrips and footrests ASTM 8.11.2, CPSC 5.3.7 - Handgrips are 3” minimum if intended to be gripped by one hand and 6” if intended to be gripped by 2 hands ASTM 8.11.2 - Seat height is between 12” and 16” for toddlers CPSC 5.3.7 - Seat height is between 14” and 28” for preschool ASTM 8.11.5, CPSC 5.3.7 - Hand grips have a diameter of 0.60” to 1.20” for toddlers CPSC 5.2.2 - Hand grips have a diameter of 0.95” to 1.55” for preschool and school age CPSC 5.2.2 - Hand grips are not protrusion hazards. ASTM 8.11 |  |